Table of Contents

# FSO Gateway Web GUI — User Guide

This document describes every screen, every panel, and every parameter in the FSO Gateway Web GUI in detail. For each field, it documents:

- Meaning — what the value represents in terms of network / FEC / system behavior.

- Source — where the value comes from: the C function, file, and line number (where relevant).

- Units — rate (Mbps / pps), time (ms / seconds), count, ratio, etc.

- How to interpret — what is normal, what thresholds apply, when to worry.

This document covers webgui-v1.3.0 (Bridge 1.3.0, telemetry schema fso-gw-stats/2).

## Table of Contents

- Architecture — three layers

- Top Bar

- Sidebar

- Dashboard

- Link Status

- Traffic

- FEC Analytics

- Interleaver

- Packet Inspector

- Channel

- Configuration

- System Logs

- Alerts

- Topology

- Analytics — recordings & history

- About

- Appendix A — Glossary

- Appendix B — Counter source map

## Architecture — three layers

The GUI does not measure anything itself. Every number you see comes from C code running as a daemon on the gateway machine. The data flow:

[Linux NIC] → [C Daemon: fso_gw_runner]
                ├─ stats_container         (atomic counters)
                ├─ deinterleaver_t          (RX FEC stats)
                ├─ arp_cache_t              (IP↔MAC table)
                └─ control_server           (AF_UNIX socket @ 10 Hz)
                          │
                          ▼  /tmp/fso_gw.sock  (ndjson, schema "fso-gw-stats/2")
                  [FastAPI Bridge: webgui/server/main.py + gateway_source.py]
                          ├─ converts C structs → browser-friendly JSON
                          ├─ computes derived values (Mbps, pps, qualityPct, alerts)
                          └─ persists history to SQLite (runs/samples)
                          │
                          ▼  WebSocket  /ws/live  (1 Hz snapshots)
                          ▼  REST       /api/*    (config, channel, runs, daemon)
                          ▼  WebSocket  /ws/logs  (stderr log stream)
                  [Next.js GUI: webgui/src/]
                          └─ browser at http://<host>:3100

Practical implications:

- The daemon measures everything in C atomics — no value is fabricated in the GUI. If a counter doesn't exist in C, the field doesn't exist in the GUI (or is explicitly flagged as Mock).

- The Bridge computes deltas over time — rates (Mbps, pps, blk/s) are computed in Python from differences between consecutive snapshots.

- The Frontend just paints — each hook (useTelemetry, useChannel, etc.) receives JSON, keeps a small in-memory history, and renders with ECharts/Tailwind.

- Mock vs. Live — if /tmp/fso_gw.sock is unavailable, the Bridge falls back to SimState (Python simulation) which produces synthetic values in the same schema. The Connection Pill in the top-left indicates "live" or "demo".

Key C files referenced throughout this guide:

| Role | File |
| --- | --- |
| Global atomic counters | src/stats.c, include/stats.h |
| Telemetry server (UNIX socket) | src/control_server.c |
| FEC RX (deinterleave + decode) | src/deinterleaver.c |
| FEC TX (fragment + encode + interleave) | src/tx_pipeline.c, src/interleaver.c |
| ARP cache (proxy-ARP) | src/arp_cache.c |
| Raw frame I/O | src/packet_io.c |
| Per-symbol CRC-32C | src/symbol.c |
| Configuration (CLI args) | src/config.c |

Bridge files (Python):

| Role | File |
| --- | --- |
| FastAPI app + WebSocket /ws/live | webgui/server/main.py |
| control_server reader, derived values, alerts | webgui/server/gateway_source.py |
| tc netem controller | webgui/server/channel.py |
| Configuration management (YAML) | webgui/server/config_store.py |
| Tail of /tmp/fso_gw.log | webgui/server/log_source.py |
| Recorder + CSV export | webgui/server/run_store.py |
| Daemon supervisor | webgui/server/daemon.py |

## Top Bar

Source: webgui/src/components/layout/TopBar.tsx

The top bar shows real-time status independent of the current page. It is sticky at the top of the screen.

### Title

| Field | Source | Notes |
| --- | --- | --- |
| FSO Gateway · Control Center | hardcoded | Fixed text |
| v3.1 · BUILD a202b70 | hardcoded | Placeholder; does not reflect the actual Bridge version. The real version is in /health and on the About page. |

### Connection Pill

| Value | Meaning |
| --- | --- |
| LIVE green | The /ws/live WebSocket is open and the Bridge is receiving data from the C daemon's socket. |
| DEMO blue | The WebSocket is open but the Bridge cannot read /tmp/fso_gw.sock — showing simulation data (SimState in main.py). |
| CONNECTING amber | The WebSocket is connecting / reconnecting (with backoff). |

The state is managed in useTelemetry.ts based on WebSocket.readyState and the source field in the payload.

### System

Shows the overall link state — not just throughput but FEC quality.

| Label | Condition | Meaning |
| --- | --- | --- |
| OPERATIONAL green | qualityPct > 99.5% | Healthy. Almost all blocks recover successfully. |
| DEGRADED amber | 95% < qualityPct ≤ 99.5% | Noticeable block losses — investigate the FSO channel. |
| OFFLINE red | qualityPct ≤ 95% | Severe state — recovery is failing. May indicate a physical disconnect. |

Computation: qualityPct = 100 × blocks_recovered / blocks_attempted. If blocks_attempted == 0 (no blocks have flowed yet), the value defaults to 100%.

Data sources:

- blocks_attempted: src/stats.c — stats_inc_block_attempt(). Called from src/deinterleaver.c when a block enters FILLING.

- blocks_recovered: stats_inc_block_success() — called when a block successfully decodes.

- The threshold logic is in gateway_source.py snapshot().

### Daemon (new in v1.3.0)

Shows the daemon state per the DaemonSupervisor in the Bridge.

| Label | Meaning |
| --- | --- |
| STOPPED gray | No active process. Click Start in the Configuration page. |
| STARTING breathing amber | Subprocess launched; the Bridge is verifying it didn't immediately exit. |
| RUNNING green | Process is alive. pid != null. |
| STOPPING breathing amber | SIGTERM sent; waiting up to 4 seconds before SIGKILL. |
| FAILED red | Process died unexpectedly, or the binary was not found. The lastError is shown on the Configuration page. |

Important note: the state reflects the supervisor in the Bridge — not the C daemon itself. If the daemon crashes without the supervisor noticing (theoretical), the watcher task will update the state to failed via proc.wait().

Source: webgui/server/daemon.py — DaemonSupervisor.status(). The hook is useDaemon which polls /api/daemon every 2 seconds.

### Uptime

Time elapsed since the daemon opened the telemetry server.

- Source: link.uptimeSec straight from uptime_sec in the snapshot. The computation uses CLOCK_MONOTONIC in src/control_server.c — elapsed_seconds().

- Format: formatUptime() in utils.ts — HhMMm or MMm SSs.

- Not the host machine / Bridge uptime — only the daemon process.

### Time (UTC)

Plain browser clock (new Date().toISOString().slice(11, 19)) updating every second. Unrelated to the daemon.

### Right-edge icons (Search / Maximize / Bell)

Cosmetic buttons. The Bell shows a red dot constantly — placeholder, not wired to the alerts feed yet.

## Sidebar

Source: webgui/src/components/layout/Sidebar.tsx

A fixed sidebar on the left with 13 navigation items. Each is a Next.js Link.

| Item | Path | Short description |
| --- | --- | --- |
| Dashboard | / | Overview: status, throughput, errors. |
| Link Status | /link-status | Quality history + fade events. |
| Traffic | /traffic | Detailed traffic: TX/RX, peak/avg, average packet size. |
| FEC Analytics | /fec-analytics | Block outcomes, burst histogram, dil_stats. |
| Interleaver | /interleaver | K×M matrix visualization, burst recoverability. |
| Packet Inspector | /packet-inspector | On-wire symbol header, CRC events. |
| Channel | /channel | tc netem control — inject loss to test FEC. |
| Configuration | /configuration | FEC parameters + Runtime Controls. |
| System Logs | /logs | Live daemon log stream. |
| Alerts | /alerts | Alert history + filtering. |
| Topology | /topology | FSO node map + ARP cache. |
| Analytics | /analytics | Recorded run list + CSV export. |
| About | /about | System status and version info. |

### Health bar at the bottom

Health 98.7% — placeholder (hardcoded). Not connected to any API. Reserved for future wiring.

## Dashboard

Source: webgui/src/app/page.tsx

Five panels updating every second via the /ws/live WebSocket.

### LinkStatusHero — Gateway diagram

Source: webgui/src/components/dashboard/LinkStatusHero.tsx

A panel with a diagram: Win-1 ── GW-A ══ GW-B ── Win-2.

| Field | Source | Notes |
| --- | --- | --- |
| LAN/FSO iface names | configEcho.lanIface, configEcho.fsoIface | Echoed straight from CLI args the daemon was launched with, via src/control_server.c (configEcho). |
| "Win-1 / Win-2" labels | hardcoded | Fixed text for the Phase 8 fixture (192.168.50.1, 192.168.50.2). |
| Status Dot green/red | derived from link.state | Green if state != "offline". |
| STATE (large text) | link.state | "ONLINE" / "DEGRADED" / "OFFLINE". |
| Quality % | link.qualityPct | Overall block-decode success rate. |
| K=N · M=N · depth=N | configEcho.k, .m, .depth | Active daemon configuration. |

### ThroughputCards — throughput tiles

Source: webgui/src/components/dashboard/ThroughputCards.tsx

Four cards: TX, RX, Packets/sec, Utilization.

#### TX Throughput

- Large value — throughput[-1].txBps formatted as Mbps/Gbps.

- Sparkline — last 60 samples of txBps.

- Sub-text — txPps (packets per second).

How the Bridge computes this:

txBps = (transmitted_bytes_now - transmitted_bytes_prev) × 8 / dt_seconds
txPps = (transmitted_packets_now - transmitted_packets_prev) / dt_seconds

C source:

- transmitted_packets, transmitted_bytes in src/stats.c — stats_inc_transmitted(size_t bytes).

- Called in src/tx_pipeline.c:554 when a symbol hits the FSO wire.

Note: transmitted counts per symbol, not per original packet. Each LAN packet is fragmented into K source symbols + M parity, and each is counted. This is the rate on the FSO wire.

#### RX Throughput

- Large value — throughput[-1].rxBps.

- Sub-text — rxPps.

Computed identically from recovered_bytes / recovered_packets.

Source:

- stats_inc_recovered(size_t bytes) in src/stats.c.

- Called in src/rx_pipeline.c (~lines 463, 502, 529) when a packet has been reassembled and forwarded out the LAN.

Note: recovered counts whole reassembled packets, not symbols. If a block fails to decode, none of its packets are counted. This means rxBps measures traffic delivered to the customer on the other side, not just bytes ingested by the daemon.

#### Packets/sec

- Large value — txPps.

- Sub-text — rxPps.

#### Link Utilization

- Large value — (latestTxBps + latestRxBps) / (10 Gbps × 2) × 100.

- Client-side computation only, assumes a 10 Gbps link in each direction.

- Tone: green <80%, amber ≥80%.

- Not based on actual NIC physical capacity — purely a conventional ratio against an assumed cap.

### ErrorMetrics — error metric cards

Source: webgui/src/components/dashboard/ErrorMetrics.tsx

Five cards across the row.

#### Symbol Loss Ratio

- Meaning: percentage of symbols that were lost on the FSO channel and never reached the deinterleaver.

- Computation: lost_symbols / total_symbols.

- C source: stats_record_symbol(bool lost) in src/stats.c lines ~441-476. Called in src/rx_pipeline.c once per symbol the daemon attempts to parse.

- Display: scientific notation when ratio < 1e-4 (e.g. 5.2e-5); percentage otherwise.

- Thresholds:

  - < 0.01% — clean channel.

  - 0.01–1% — moisture/particles. Still within FEC budget.

  - > 5% — failures will start.

#### Block Fail Rate

- Meaning: percentage of blocks that failed to decode entirely.

- Computation: blocks_failed / blocks_attempted.

- Source: stats_inc_block_failure() is called from src/deinterleaver.c for: timeout (insufficient symbols arrived in time), too-many-holes (more than M holes).

- Tone: green at 0%, amber >0.1%, red >0.1%.

#### CRC Drops

- Meaning: number of symbols rejected by the per-symbol CRC-32C check (when enabled).

- Source: stats_inc_crc_drop_symbol() in src/stats.c line ~485. Called in src/rx_pipeline.c or src/symbol.c when CRC mismatch.

- Use: a CRC-failed symbol is treated as an unknown erasure (FEC may substitute it if enough redundancy exists).

- When to worry: if the count climbs rapidly, the link is suffering corruption (not just loss).

#### Recovered Packets

- Meaning: cumulative count of LAN packets reassembled and forwarded out.

- Source: recovered_packets in src/stats.c, stats_inc_recovered().

- Call sites: src/rx_pipeline.c lines 463, 502, 529 (three reassembly success paths).

#### FEC Success Rate

- Computation: (blocksRecovered / blocksAttempted) × 100.

- Computed client-side (from the same errors fields in the snapshot).

- Tone: green >99.9%, cyan >99%, amber otherwise.

### LiveCharts — real-time charts

Source: webgui/src/components/dashboard/LiveCharts.tsx

#### Throughput Over Time

- Two series: TX Mbps (cyan) and RX Mbps (blue dashed).

- Last 180 samples (3 minutes at 1 Hz).

- Y-axis in Mbps; X-axis is relative time.

#### Packet Rate (pps)

- Two series: TX pps, RX pps.

- Last 180 samples.

#### Burst-Length Distribution

- Histogram of 7 bins: 1, 2-5, 6-10, 11-50, 51-100, 101-500, 501+.

- C source: burst_len_1 ... burst_len_501_plus in src/stats.c lines 50-56.

- How a burst is computed: stats_record_symbol() counts a run of consecutive lost symbols. When a non-lost symbol arrives, the burst closes and is classified into the appropriate bin in stats_close_current_burst() (src/stats.c lines 99-115).

- Colors: cyan/blue for bins recoverable within M × depth, amber/red for those exceeding.

#### Cumulative Counters

- Blocks with loss — decoderStress.blocksWithLoss ← blocks_with_loss in src/stats.c line 66. Blocks that had at least one hole post-deinterleave.

- Total holes — decoderStress.totalHolesInBlocks ← sum of all holes across all blocks.

- Recoverable bursts — recoverable_bursts in src/stats.c line 60. Bursts whose length ≤ M × depth.

- Critical bursts — critical_bursts in src/stats.c line 61. Bursts that exceeded the span and could not have been recovered.

## Link Status

Source: webgui/src/app/link-status/page.tsx

### Hero Tiles

| Field | Source | Notes |
| --- | --- | --- |
| STATE | link.state | "ONLINE" / "DEGRADED" / "OFFLINE" — derived in the Bridge. |
| Quality % | link.qualityPct | (blocks_recovered / blocks_attempted) × 100. |
| Symbol Loss | errors.symbolLossRatio | Ratio; shown as percentage or 1e-X. |
| Block Fail Rate | errors.blockFailRatio | blocks_failed / blocks_attempted. |
| Blocks Attempted | errors.blocksAttempted | Cumulative since daemon launch. |
| Session Uptime | link.uptimeSec | Same source as the TopBar. |

### Quality Over Time

A line chart of qualityPct over time. Y-axis range 80–100% to amplify resolution near the top.

- Source: useLinkHistory reads each snapshot and stores history client-side (not persisted server-side).

- Window: up to several hundred samples; lives in browser memory only.

### Symbol Loss Ratio

A logarithmic chart of symbol-loss ratio over time.

- Log Y-axis to span a wide range (1e-6 to 1.0).

- Same data source as the Quality sample.

### Stability Panel

| Field | Source | Note |
| --- | --- | --- |
| Session Uptime % | client computation | Ratio of time spent in online/degraded vs. total observed time. |
| Observed For | client computation | How long the page has been open. |
| In Current State | client computation | Time since the last state transition. |
| Fade Events | client computation | Number of online↔degraded↔offline transitions detected. |

⚠️ The entire panel is client-side analysis of the WebSocket history. There is no "fade detection" mechanism in the C daemon — it's analytics done in the browser.

### Fade Timeline

A table with one row per fade event:

- Timestamp — tStart in ms.

- From → To — state transition (online → degraded).

- Lowest Quality — minimum qualityPct observed during the fade.

- Duration — tEnd - tStart, or "ongoing".

⚠️ Synthetic in the same way.

## Traffic

Source: webgui/src/app/traffic/page.tsx

A more detailed traffic page — uses the same fields as the Dashboard.

### TX Card / RX Card

Per card:

| Field | Source | Notes |
| --- | --- | --- |
| Current Rate | throughput[-1].txBps / .rxBps | Current rate in Mbps/Gbps. |
| Current PPS | throughput[-1].txPps / .rxPps | Packets per second. |
| Utilization | (latestBps / 10e9) × 100 | Relative to 10 Gbps (arbitrary assumption). |
| Peak Rate | max(history.txBps) | Maximum across the available history. |
| Avg Rate | avg(history.txBps) | Average. |
| Peak PPS | max(history.txPps) | Peak packet rate. |
| Avg Packet Size | latestBps / 8 / latestPps | Bytes-per-packet computation. |
| Sparkline | throughput.slice(-60) | 60 samples (1 minute). |

History is held in the client (useTelemetry keeps up to 300 samples). There is no separate Traffic endpoint — everything comes in the single snapshot.

### Charts

- Throughput — two series txBps/rxBps over 300 samples.

- PPS — two series txPps/rxPps.

### Summary Metrics

| Field | Computation |
| --- | --- |
| Combined Throughput | latestTxBps + latestRxBps. |
| Combined PPS | latestTxPps + latestRxPps. |
| Avg TX Packet | txBps / 8 / txPps. |
| Peak Utilization | max(peakTxBps, peakRxBps) / 10e9 × 100. |

## FEC Analytics

Source: webgui/src/app/fec-analytics/page.tsx

This is the most important page for understanding FEC behavior.

### Hero Metrics

| Field | Source | Notes |
| --- | --- | --- |
| Recovery Rate | (blocksRecovered / blocksAttempted) × 100 | Same as Quality elsewhere; shown in FEC context. |
| Blocks/sec | average of last 5 samples | Block decode rate. Sub-text: recovered/sec and failed/sec. |
| Failed Blocks | errors.blocksFailed | Cumulative. Tone by failure rate (>0.1% = warning). |
| Symbol Loss | errors.symbolLossRatio | Same as elsewhere. |
| Critical Bursts | sum of 101-500 and 501+ bins of burstHistogram | Cases the FEC could not have covered. |

### FEC Config Panel

Shows the daemon's active configuration.

| Field | Source |
| --- | --- |
| K (Source Symbols) | configEcho.k — origin: daemon CLI flag --k, echoed back in the snapshot. |
| M (Repair Symbols) | configEcho.m — --m. |
| Overhead | m / (k + m) × 100 — client-side. |
| Code Rate | k / (k + m) — client-side. |
| LAN Interface | configEcho.lanIface — --lan-iface. |
| FSO Interface | configEcho.fsoIface — --fso-iface. |
| Symbol Size | configEcho.symbolSize — --symbol-size. |
| Symbol CRC | configEcho.internalSymbolCrc — "CRC-32C" if true, "Disabled" otherwise. |

Why surface the config on every page? Because these parameters affect the meaning of every other number. A burst of length 50 is critical when M × depth = 32, but harmless when M × depth = 100.

### Block Outcomes Over Time

A chart with two stacked series:

- Recovered blocks/sec — per-second delta of blocks_recovered.

- Failed blocks/sec — per-second delta of blocks_failed.

Source: useFecHistory reads each snapshot and stores history.

### Burst-Length Distribution

Same as Dashboard, but with a logarithmic Y-axis (wider range).

### Deinterleaver Panel (important!)

This is the only place in the GUI showing the deinterleaver's internal dil_stats — added in the fso-gw-stats/2 schema.

| Field | C source | Meaning |
| --- | --- | --- |
| Active | dil_stats.active_blocks in src/deinterleaver.c | Blocks in FILLING or READY_TO_DECODE — current occupancy. |
| Ready | dil_stats.ready_count | Blocks completed and waiting on deinterleaver_get_ready_block(). |
| Blocks ready (cumulative) | dil_stats.blocks_ready | Cumulative since launch. |
| Failed · timeout | dil_stats.blocks_failed_timeout | Blocks discarded because not enough symbols arrived in time (timeout = 50 ms by default). |
| Failed · holes | dil_stats.blocks_failed_holes | Blocks that could not be decoded due to more than M holes. |
| Dropped · duplicate | dil_stats.dropped_symbols_duplicate | A symbol arrived twice (same fec_id). |
| Dropped · frozen | dil_stats.dropped_symbols_frozen | A symbol arrived for a block that has already left FILLING (too late). |
| Dropped · erasure | dil_stats.dropped_symbols_erasure | Symbols marked is_erasure=1 (lost). |
| Dropped · CRC fail | dil_stats.dropped_symbols_crc_fail | Symbols rejected by CRC-32C. |
| Evicted · filling | dil_stats.evicted_filling_blocks | Slot was evicted while still in FILLING (out of buffer space). |
| Evicted · done | dil_stats.evicted_done_blocks | Slot was evicted from READY_TO_DECODE without being drained. |

What causes eviction? The deinterleaver is implemented as a circular buffer. When more new blocks arrive than the capacity (depth × 4), older blocks are pushed out. This shouldn't happen in production — if it does, there is RX backpressure.

### Block Lifecycle Event Feed

A table of recent block events. Shows every transition in the deinterleaver FSM.

| Column | Source |
| --- | --- |
| Timestamp | blockEvents[].t — now_ms() in src/control_server.c. |
| Reason | blockEvents[].reason — enum mapped from deinterleaver_block_final_reason_t. Possible values: SUCCESS, DECODE_FAILED, TIMEOUT, TOO_MANY_HOLES, EVICTED_FILLING, EVICTED_READY. |
| Block ID | blockEvents[].blockId — from the block itself. |
| Evicted | blockEvents[].evicted — bool, true if the block closed via eviction. |

Behind the scenes: the deinterleaver registers a callback in the control_server, and every final transition is pushed to a list (block_events ring buffer of ~128 entries). The Bridge surfaces this in the snapshot.

### Decoder Stress Panel

| Field | Source |
| --- | --- |
| Blocks with Loss | blocks_with_loss — counted when a block finalized with at least one hole. |
| Worst Holes / Block | worst_holes_in_block — maximum holes observed in any single block. |
| Total Holes | total_holes_in_blocks — sum across all blocks. |
| Recoverable Bursts | recoverable_bursts — burst length ≤ M × depth. |
| Critical Bursts | critical_bursts — bursts that exceeded. |
| Exceeding FEC Span | bursts_exceeding_fec_span — usually identical to Critical (differs in edge cases). |

General source: src/stats.c lines 58-68. The classification happens in stats_close_current_burst() (lines 137-164).

## Interleaver

Source: webgui/src/app/interleaver/page.tsx

Largely a documentation page — illustrates the matrix structure.

### Metric Cards

| Field | Computation | Notes |
| --- | --- | --- |
| Matrix (depth × K+M) | configEcho.depth × (k + m) | Matrix dimensions. |
| Cell count | depth × (k + m) | Total matrix cells. |
| Matrix Size | cell_count × symbolSize | Byte size. |
| Recovery Span | m × depth | Maximum number of consecutive lost symbols still recoverable. |
| Burst Coverage | (burstWithinSpan / totalBursts) × 100 | Percentage of observed bursts that were recoverable. |
| Exceeding Span | decoderStress.burstsExceedingFecSpan | Bursts too long to recover. |

### Matrix Layout (visual)

An interactive visualization of the matrix (depth rows × K+M columns):

- The first K columns are colored cyan (data).

- The last M columns are colored amber (parity).

### Derivations Panel

Repeats the same parameters as a list:

| Field | Computation |
| --- | --- |
| Depth (rows) | configEcho.depth |
| Block width | k + m |
| Total symbols / block | k + m |
| Max burst recoverable | m × depth |
| Symbol size | configEcho.symbolSize |
| Block size | (k + m) × symbolSize |

## Packet Inspector

Source: webgui/src/app/packet-inspector/page.tsx

A page explaining the on-wire symbol format plus activity metrics.

### Activity Cards

| Field | Source | Notes |
| --- | --- | --- |
| Packets/sec (TX) | throughput[-1].txPps | Same as Traffic. |
| RX packets/sec | throughput[-1].rxPps | Sub-text. |
| Avg Packet Size | latestBps / 8 / latestPps | Bytes. |
| Symbols each (approx) | avgPacketSize / symbolSize | How many symbols per packet. |
| Symbols Processed | blocksAttempted × k | Estimate of total symbols processed. |
| CRC Drops | errors.crcDrops | Total CRC failures. |
| CRC enabled/disabled | configEcho.internalSymbolCrc | Descriptive text. |

### Wire Format Diagram

Shows the symbol header layout — 18 fixed bytes:

| Field | Bytes | C source |
| --- | --- | --- |
| packet_id | 4 | include/symbol.h |
| fec_id | 4 | The symbol's index within the block (0..K-1 = source, K..K+M-1 = parity). |
| symbol_index | 2 | Position within the original block. |
| total_symbols | 2 | K+M. |
| payload_len | 2 | Data length. |
| crc32 | 4 | CRC-32C over the payload (when enabled). |
| payload | symbolSize | Actual data (typically 800-1500 bytes). |

### Recent Symbol Events

A logs table filtered to symbol events (keywords: pkt_id, block_id, symbols, fragment, reassemble).

Source: WebSocket /ws/logs connected via useLogs.

⚠️ When the daemon is not running, the Bridge generates synthetic logs — not real.

## Channel

Source: webgui/src/app/channel/page.tsx and useChannel.ts

A page that uses Linux tc netem to inject packet loss into the FSO channel for FEC testing.

### Presets

Six presets that load potential slider values. They are not applied automatically — you must click Apply.

| Preset | enterPct | exitPct | lossPct | Description |
| --- | --- | --- | --- | --- |
| Clear | 0 | 0 | 0 | Disables netem, clean channel. |
| Drizzle | 1 | 70 | 0.5 | Sparse single-symbol loss. |
| Weather | 5 | 50 | 5 | Moderate fades, still within FEC budget. |
| Haze | 8 | 30 | 8 | Sustained low-level loss. |
| Storm | 12 | 25 | 20 | Heavy bursts, stresses FEC. |
| Blackout | 30 | 10 | 50 | Pathological loss, expected to fail. |

### Sliders

| Slider | Meaning | Units |
| --- | --- | --- |
| Enter % (Burst Entry) | Probability of transitioning from "good" to "bad" Gilbert-Elliott state. | Percent 0-100. |
| Exit % (Burst Exit) | Probability of leaving the "bad" state. | Percent 0-100. |
| Loss % (in bad state) | Packet loss rate while in the "bad" state. | Percent 0-100. |

### Buttons

- Apply — sends a POST to /api/channel/netem with the values. The Bridge runs:

- tc qdisc replace dev <FSO_iface> root netem loss gemodel <enter>% <exit>% <loss>%

- Source: webgui/server/channel.py — apply_gemodel().

- Clear — sends a POST with {clear: true}. The Bridge runs tc qdisc del.

### Status Display

| Field | Source | Notes |
| --- | --- | --- |
| iface | local state, default enp1s0f1np1 | Editable. |
| available | result of tc qdisc show | Whether the tc binary is reachable (checked on the iface). |
| active | Whether a netem qdisc is currently active. |  |
| model | "gemodel" / "uniform" / null | Active loss model. |
| lossPct / enterPct / exitPct | parsed from tc output | Current values from the system. |

### Permission requirements

tc qdisc add/del requires CAP_NET_ADMIN. Either the Bridge runs as root, or sudoers grants tc. If neither, the field will show available=false and a warning.

## Configuration

Source: webgui/src/app/configuration/page.tsx

Daemon configuration editor. Changes are persisted to webgui/server/config.yaml and require a restart to take effect.

### FEC Parameters Panel

| Parameter | Range | Meaning | Derived value |
| --- | --- | --- | --- |
| K — Source Symbols | 4–256 | How many original symbols per block. Larger K = less overhead, more latency. | — |
| M — Repair Symbols | 1–256 | How many extra (parity) symbols. Larger M = better recovery capability. | — |
| Overhead | derived | m / (k + m) × 100 — redundancy percentage. | M=4, K=8 → 33% overhead. |
| Code Rate | derived | k / (k + m) — information efficiency. | M=4, K=8 → 0.667. |
| Block Size | derived | (k + m) × symbolSize bytes. | K=8, M=4, sym=1500 → 18000 bytes. |
| Burst Recovery ~ | derived | m × depth — maximum recoverable burst length. | M=4, depth=16 → 64 symbols. |

### Interleaver Panel

| Parameter | Range | Meaning |
| --- | --- | --- |
| Depth (rows) | 1–256 | Number of rows in the interleaver matrix. Higher depth = better burst spread, but more latency and memory. |

### Symbol Panel

| Parameter | Range | Meaning |
| --- | --- | --- |
| Symbol Size | 64–4096 | Payload bytes per symbol. Larger = less overhead, but higher exposure to single-symbol loss. |

### Network Interfaces Panel

| Parameter | Meaning |
| --- | --- |
| LAN Interface | The interface for client traffic. The daemon will pcap_open and capture frames. |
| FSO Interface | The interface for the FSO link. Encoded symbols are transmitted through it. |

### Symbol Integrity Panel

| Parameter | Meaning |
| --- | --- |
| Internal Symbol CRC-32C | When enabled, every symbol carries a CRC-32C over the payload. On RX, mismatched CRC drops the symbol before the FEC layer sees it. Effect: higher robustness against in-channel corruption; small throughput cost (4 bytes/symbol). |

### Action Bar

| Button | Action |
| --- | --- |
| Apply Changes | Sends a POST /api/config with the draft. The Bridge validates via config_store.validate() and writes YAML. A daemon restart is required to take effect. |
| Revert | Reverts the client-side draft. |

### Live Derived

The same derived values shown in the right column — read-only.

### Runtime Controls (Phase 3B)

Source: useDaemon and webgui/server/daemon.py

| Button | Action |
| --- | --- |
| Start Gateway | POST /api/daemon/start — the supervisor performs subprocess.Popen with argv built from config.yaml. |
| Restart | POST /api/daemon/restart — Stop then Start. |
| Stop | POST /api/daemon/stop — sends SIGTERM, then SIGKILL after 4s. |

Detailed status display:

| Field | Notes |
| --- | --- |
| PID | Subprocess identifier. |
| Uptime | Time since start. |
| Sudo | Whether the supervisor prepends sudo -n (per FSO_DAEMON_SUDO=1). |
| Binary | "found"/"missing" per os.access(path, X_OK). |
| binary path | Path to the binary (FSO_DAEMON_BINARY or default fso_gw_runner). |
| log | Log file (FSO_DAEMON_LOG, default /tmp/fso_gw.log). |

### Daemon State Badge

See TopBar > Daemon — same logic.

### Relevant environment variables

| Variable | Default | Meaning |
| --- | --- | --- |
| FSO_DAEMON_BINARY | fso_gw_runner | Path to the binary to launch. Can point to control_server_demo for development. |
| FSO_DAEMON_SUDO | 0 | If 1, prepends sudo -n to argv (typically required in production for raw pcap). |
| FSO_DAEMON_LOG | /tmp/fso_gw.log | Where stdout+stderr are redirected. |
| FSO_DAEMON_KILL_ON_EXIT | 0 | If 1, the Bridge stops the daemon on its own shutdown (otherwise the daemon outlives the bridge). |

## System Logs

Source: webgui/src/app/logs/page.tsx and useLogs.ts

Live log stream from the daemon.

### Log Console

| Field | Source |
| --- | --- |
| Live stream | WebSocket /ws/logs. |
| Timestamp | event.ts_ms — from daemon stderr, or time.time() if generated. |
| Level | event.level — DEBUG/INFO/WARN/ERROR per src/logging.c. |
| Module | event.module — subsystem (e.g. tx_pipeline, deinterleaver). |
| Message | event.message — text. |

### Controls

| Control | Purpose |
| --- | --- |
| Level filter | Filter by severity. |
| Module filter | Filter to a subsystem. |
| Search | Substring search in any field. |
| Pause / Resume | Halts rendering (the server keeps streaming). |
| Export | ⚠️ Not implemented yet. |

### Data source

The Bridge tails /tmp/fso_gw.log (or FSO_LOG_FILE) in log_source.py. If there is no file, it generates synthetic logs (e.g. "Mock log entry from sim_runner.c") — flagged with source: "mock" in each event.

### Log levels in C

Source: include/logging.h:

typedef enum {
    LOG_DEBUG, LOG_INFO, LOG_WARN, LOG_ERROR
} log_level_t;

Filtering happens in log_set_level(). Production default: INFO.

## Alerts

Source: webgui/src/app/alerts/page.tsx and useAlerts.ts

### Alert tabs

- Active — alerts the user has not acknowledged.

- Acknowledged — alerts the user has dismissed (stored in localStorage).

- All — everything.

### Filtering

- Severity checkboxes — critical / warning / info.

- Module dropdown — LINK / FEC / CRC / BURST / ARP / CONFIG / *.

- Search — substring.

### Alert Event Details

| Field | Source |
| --- | --- |
| ID | alert.id — {ts}-{module}-{counter}, generated in the Bridge. |
| Timestamp | alert.t — int(time.time() * 1000) in gateway_source.py. |
| Severity | alert.severity — one of critical/warning/info. |
| Module | alert.module — subsystem. |
| Message | alert.message — human-readable text. |
| Acknowledged | localStorage — client-side only. |

### Alert generation rules (Live)

Source: gateway_source.py — _detect_alerts(). Called for each incoming snapshot, comparing to the previous one.

| Condition | Severity | Module | Message |
| --- | --- | --- | --- |
| delta(blocks_failed) >= 3 | critical | FEC | "{N} FEC blocks failed this tick" |
| delta(symbols_dropped_crc) >= 20 | warning | CRC | "{N} symbols dropped on CRC" |
| max_burst_length increased and exceeded 20 | warning | BURST | "Max burst length climbed to {N}" |
| delta(bursts_exceeding_fec_span) > 0 | critical | BURST | "{N} burst(s) exceeded FEC span this tick" |

### Mock alerts (when the daemon is not running)

In main.py, _maybe_emit_alert() produces alerts at ~15% probability per tick, drawn from arbitrary templates. Demo only.

### Ring buffer

The list in the Bridge is capped at ALERT_RING_MAX (~200 entries). The GUI receives only the most recent in each snapshot.

## Topology

Source: webgui/src/app/topology/page.tsx

### Physical Layout

A fixed diagram (Phase 8 fixture):

Win-1 (192.168.50.1) ──[LAN]── GW-A ══[FSO cable]══ GW-B ──[LAN]── Win-2 (192.168.50.2)

### Static Labels

- GW-A / GW-B — fixed.

- Win-1 · 192.168.50.1, Win-2 · 192.168.50.2 — fixed.

- LAN/FSO iface names — from configEcho.lanIface/.fsoIface.

### FsoBeam Visualization

An animated component (Framer Motion) simulating the light beam.

| Property | Source | Computation |
| --- | --- | --- |
| Beam color | link.state | green/amber/red. |
| Quality % label | link.qualityPct | Shown inside the beam. |
| TX intensity | min(1, txPps / 5000) | Particle density (animation). |
| RX intensity | min(1, rxPps / 5000) | Same. |
| TX/RX pps labels | throughput[-1].txPps/.rxPps | "TX 1234 pps". |

### ARP Cache Panel

A table of entries learned by the daemon's arp_cache.

| Column | Source |
| --- | --- |
| IP | arpEntries[].ip — from src/arp_cache.c. |
| MAC | arpEntries[].mac. |
| Last Seen | now - arpEntries[].lastSeenMs — client-side, e.g. "5s ago". |

Who writes to the table?

- In fso_gw_runner — arp_cache_learn() is called in src/rx_pipeline.c when an ARP request's src_ip is recognized from the LAN side.

- In control_server_demo — seed_arp_cache() calls arp_cache_learn() with the fixed Phase 8 entries every 5 seconds (because the TTL is 5 minutes).

The snapshot includes the table via arp_cache_get_entries(), which returns struct arp_entry[].

⚠️ In mock mode (Bridge with no daemon), the table is a fixed list of 2 entries.

### Recent Link Events

A table identical to the one in Alerts — but filtered to the most recent events.

## Analytics — recordings & history

Source: webgui/src/app/analytics/page.tsx and useRuns.ts

A page for recording and comparing historical runs. Data is persisted in SQLite (webgui/server/runs.db).

### Runs Tab

#### Active Run

| Field | Source |
| --- | --- |
| activeRunId | useRuns().activeRunId ← /health or /api/runs. |
| New Run | POST /api/runs/new → run_store.create_run(). Creates a new record and starts sampling at 1 Hz. |

#### Runs List

A table of all runs:

| Column | Source |
| --- | --- |
| ID | run.id — auto-increment in SQLite. |
| Name | run.name — optional, editable. |
| Start time | run.start_ms — int(time.time() * 1000) at creation. |
| End time | run.end_ms — null if active. |
| Duration | (end_ms - start_ms) / 1000. |
| Sample count | run.sample_count. |

#### Run Detail

You can pick a run and view:

| Chart | Source |
| --- | --- |
| Throughput over time | samples[].txBps/rxBps — sampled at 1 Hz. |
| Quality (FEC) over time | samples[].qualityPct. |
| FEC block outcome rate | recovered/failed per second. |

Sampled by _recorder_loop() in main.py — a loop that takes a snapshot every second and writes to SQLite.

#### Summary Stats

| Field | Computation |
| --- | --- |
| Peak/Avg throughput | max/avg of txBps/rxBps across all samples. |
| Min/Max/Avg quality | Statistics over qualityPct. |
| Total/Recovered/Failed blocks | From the run's last snapshot. |

#### Export CSV

GET /api/runs/{runId}/export.csv — returns a CSV with columns t,txBps,rxBps,txPps,rxPps,qualityPct,.... Built in run_store.py — export_csv().

### Experiments Tab

A placeholder page. The API exists (/api/experiments in experiments.py) — reads .txt files from /tmp/fso_experiments/ and parses them. The UI does not yet render them in detail.

## About

Source: webgui/src/app/about/page.tsx

A meta page about the system.

### System Status Cards

| Card | Field | Source | Notes |
| --- | --- | --- | --- |
| Bridge | "Connected" / "Unreachable" | useHealth() — GET /health. | Whether FastAPI responds. |
|  | "Tick {Hz}" | health.tick_hz | Bridge sampling rate (default 1 Hz). |
| Gateway | "Live (control_server)" / "Mock data" | health.source | "gateway" if the socket is up, "mock" otherwise. |
|  | "Streaming via UNIX socket" | health.source == "gateway" | Status indicator. |
| Recording | "Run #{id}" / "Not recording" | health.active_run_id | Recorder state. |
|  | logs mode | health.logs_mode | "live"/"tail"/"file"/"idle". |

### Architecture Diagram

A static diagram (not dynamic) of the three layers:

Browser  ─[WS/REST :8000]─→  FastAPI Bridge  ─[UNIX socket]─→  C Daemon

### Tech Stack

A fixed list of libraries and versions.

### Roadmap

A table of phases and status:

| Phase | Status |
| --- | --- |
| Phase 1 — Shell + design | Done |
| Phase 2 — FastAPI bridge + control_server | Done |
| Phase 3A — Config persistence | Done |
| Phase 3B — Daemon supervision | Done (v1.3.0) |
| Phase 4 — Feature pages | Done |
| Phase 5 — Analytics + CSV | Done |
| Polish — Cosmetic pass | Pending |

## Appendix A — Glossary

| Term | Definition |
| --- | --- |
| FEC (Forward Error Correction) | Sending more data than strictly required so the receiver can correct losses without asking for retransmission. |
| Wirehair | A specific fountain code library — takes K source symbols and produces K+M output symbols. The decoder can reconstruct from any K of them. |
| Block | A logical unit of K source + M parity symbols = K+M total. |
| Symbol | The atomic unit on the wire. Each symbol carries an 18-byte header + payload (symbolSize). |
| Burst | A run of consecutive lost symbols. The FEC withstands bursts up to M × depth. |
| Interleaver | A matrix (depth rows × K+M columns) that scrambles symbols during transmission so symbols from the same block do not appear consecutively on the wire — a physical loss burst spreads across multiple blocks rather than ruining one. |
| Deinterleaver | The receiver-side counterpart — it collects symbols off the wire and restores them to their original blocks. Responsible for evicting late symbols. |
| CRC-32C (Castagnoli) | Polynomial 0x82F63B78. A 4-byte check over each symbol's payload. When enabled, mismatched symbols are dropped as erasures. |
| AF_UNIX socket | A local file-based socket. The daemon's telemetry server opens one at /tmp/fso_gw.sock. |
| WebSocket | A bidirectional protocol layered over HTTP. The GUI connects to the Bridge's /ws/live and receives snapshots. |
| netem | The Linux Network Emulator. A qdisc component that injects latency, loss, reorder, etc. The GUI uses it via tc to inject loss. |
| Gilbert-Elliott model | A two-state loss model ("good"/"bad") with transition probabilities between them. Produces more realistic bursts than uniform random loss. |
| proxy-ARP | A technique where the daemon answers ARP requests for IPs across the link, allowing client LANs to communicate without explicit routing. |

## Appendix B — Counter source map

A quick reference: every value visible in the GUI, mapped to its origin in C.

### TX side

| GUI field | C counter | File:line | Function |
| --- | --- | --- | --- |
| TX throughput (Mbps/pps) | transmitted_packets, transmitted_bytes | src/stats.c | stats_inc_transmitted() |
|  |  | src/tx_pipeline.c:554 | call site at FSO TX |
| ingress (not directly displayed; part of stats) | ingress_packets, ingress_bytes | src/stats.c | stats_inc_ingress() |
|  |  | src/tx_pipeline.c:311 | call site when packet enters from LAN |

### RX side

| GUI field | C counter | File:line | Function |
| --- | --- | --- | --- |
| RX throughput | recovered_packets, recovered_bytes | src/stats.c | stats_inc_recovered() |
| Recovered Packets | same | src/rx_pipeline.c:463,502,529 | after successful reassembly |
| Failed Packets | failed_packets | src/stats.c | stats_inc_failed_packet() |

### Symbols

| GUI field | C counter | File:line |
| --- | --- | --- |
| Symbol Loss Ratio | lost_symbols / total_symbols | src/stats.c:441-476 — stats_record_symbol(bool lost) |
| CRC Drops | symbols_dropped_crc | src/stats.c:485 — stats_inc_crc_drop_symbol() |

### Blocks (FEC)

| GUI field | C counter | File:line |
| --- | --- | --- |
| Blocks Attempted | blocks_attempted | src/stats.c:470-474 — stats_inc_block_attempt() |
| Blocks Recovered | blocks_recovered | src/stats.c:475-479 — stats_inc_block_success() |
| Blocks Failed | blocks_failed | src/stats.c:480-484 — stats_inc_block_failure() |
| Quality % | computed: blocks_recovered / blocks_attempted × 100 | gateway_source.py snapshot() |

### Bursts

| GUI field | C counter | File:line |
| --- | --- | --- |
| Burst histogram (7 bins) | burst_len_1, _2_5, _6_10, _11_50, _51_100, _101_500, _501_plus | src/stats.c:50-56 |
| Recoverable Bursts | recoverable_bursts | src/stats.c:60, line 162 |
| Critical Bursts | critical_bursts | src/stats.c:61, line 157 |
| Exceeding FEC Span | bursts_exceeding_fec_span | src/stats.c:58, line 158 |
| Max Burst Length | max_burst_length | src/stats.c — updated in stats_close_current_burst() |
| Total Bursts | total_bursts | src/stats.c |

### Decoder Stress

| GUI field | C counter | File:line |
| --- | --- | --- |
| Blocks with Loss | blocks_with_loss | src/stats.c:66 — stats_record_block(holes) |
| Total Holes | total_holes_in_blocks | src/stats.c:68 |
| Worst Holes / Block | worst_holes_in_block | src/stats.c:68 — atomic max |

### Deinterleaver Stats (dil_stats)

Source: src/deinterleaver.c, struct in include/deinterleaver.h.

| GUI field | C field | When updated |
| --- | --- | --- |
| Active | active_blocks | Blocks in FILLING + READY_TO_DECODE. |
| Ready | ready_count | Blocks in READY_TO_DECODE only. |
| Blocks ready (cumulative) | blocks_ready | Cumulative. |
| Failed timeout | blocks_failed_timeout | Block left because not enough symbols arrived in 50 ms. |
| Failed holes | blocks_failed_holes | Block left because >M holes. |
| Dropped duplicate | dropped_symbols_duplicate | Same fec_id arrived twice. |
| Dropped frozen | dropped_symbols_frozen | Symbol arrived for a block not in FILLING. |
| Dropped erasure | dropped_symbols_erasure | Symbol with is_erasure=1. |
| Dropped CRC fail | dropped_symbols_crc_fail | CRC failed. |
| Evicted filling | evicted_filling_blocks | Slot evicted in FILLING. |
| Evicted done | evicted_done_blocks | Slot evicted in READY_TO_DECODE. |

### ARP Cache

Source: src/arp_cache.c, struct in include/arp_cache.h.

| GUI field | C field |
| --- | --- |
| ip / mac | struct arp_entry { uint32_t ip; uint8_t mac[6]; uint64_t last_seen_ms; } |

### Block Events (FEC Lifecycle)

Source: callbacks from src/deinterleaver.c → ring buffer in src/control_server.c.

| GUI reason | C enum |
| --- | --- |
| SUCCESS | DIL_BLOCK_FINAL_REASON_SUCCESS |
| DECODE_FAILED | DIL_BLOCK_FINAL_REASON_DECODE_FAILED |
| TIMEOUT | DIL_BLOCK_FINAL_REASON_TIMEOUT |
| TOO_MANY_HOLES | DIL_BLOCK_FINAL_REASON_TOO_MANY_HOLES |
| EVICTED_FILLING | DIL_BLOCK_FINAL_REASON_EVICTED_FILLING |
| EVICTED_READY | DIL_BLOCK_FINAL_REASON_EVICTED_READY |

### Config Echo

| GUI field | C source | CLI flag |
| --- | --- | --- |
| configEcho.k | cfg.k | --k |
| configEcho.m | cfg.m | --m |
| configEcho.depth | cfg.depth | --depth |
| configEcho.symbolSize | cfg.symbol_size | --symbol-size |
| configEcho.lanIface | cfg.lan_iface | --lan-iface |
| configEcho.fsoIface | cfg.fso_iface | --fso-iface |
| configEcho.internalSymbolCrc | cfg.internal_symbol_crc_enabled | --internal-symbol-crc 1 |

The config is passed to control_server in gateway_create() (src/gateway.c) and is included in every snapshot built by src/control_server.c build_snapshot_json().

### Computed fields (not in C)

| GUI field | Computation | Where |
| --- | --- | --- |
| Quality % | 100 × blocks_recovered / blocks_attempted | gateway_source.py snapshot() |
| txBps / rxBps | (bytes_delta × 8) / dt_seconds | gateway_source.py — _ingest() |
| txPps / rxPps | packets_delta / dt_seconds | same |
| Symbol Loss Ratio | lost_symbols / total_symbols | gateway_source.py snapshot() |
| Block Fail Rate | blocks_failed / blocks_attempted | same |
| FEC Success Rate | (blocks_recovered / blocks_attempted) × 100 | client-side in ErrorMetrics.tsx |
| Link Utilization | (txBps + rxBps) / 20Gbps × 100 | client-side, fixed 10Gbps per direction |
| Burst Coverage | burstWithinSpan / totalBursts × 100 | client-side on the Interleaver page |
| Avg Packet Size | txBps / 8 / txPps | client-side |
| Symbols each (approx) | avgPacketSize / symbolSize | client-side |

### Mock / Fabricated fields

⚠️ The following are not backed by C counters — either computed in the browser or simulated:

| Field | Where | Origin |
| --- | --- | --- |
| Daemon state | TopBar, Configuration | DaemonSupervisor in the Bridge — reflects supervisor intent, not the C process. |
| Fade events (Link Status) | Link Status page | Client-side computation on quality history. |
| Sidebar Health 98.7% | Sidebar | hardcoded. |
| Notification badge | TopBar | hardcoded. |
| Mock alerts | Alerts page (when daemon is down) | random in main.py:_maybe_emit_alert(). |
| Mock logs | Logs page (when daemon is down) | generated in log_source.py. |
| Mock ARP entries | Topology (when daemon is down) | fixed list in main.py. |

## Common troubleshooting

### "Connection: DEMO" while the daemon is running

- Check that the socket exists: ls -la /tmp/fso_gw.sock.

- Check the Bridge is connecting: tail -f /tmp/bridge.log and look for "gateway connected".

- Make sure no other process is binding the same socket path.

### "Daemon: FAILED" with "binary not found"

- Check FSO_DAEMON_BINARY in the Bridge's environment.

- Verify the file exists and has +x.

- On Linux you may need setcap cap_net_raw,cap_net_admin=eip on the binary — or run with sudo.

### "Daemon: FAILED" with "exited immediately rc=N"

- Manually run the same argv shown in status.command to see the stderr error.

- For fso_gw_runner a common cause: the iface does not exist, or the process lacks permissions for pcap_open.

### Channel page errors ("tc: command not found" / "permission denied")

- Verify tc is on the Bridge's PATH.

- For the FSO iface: either run the Bridge as root, or add a sudoers entry like NOPASSWD: /sbin/tc.

### Throughput charts show zero

- Confirm there is real traffic on LAN. Run iperf3 from Win-1 to Win-2 to verify.

- In demo mode (control_server_demo) the throughput rate is synthetic and independent of real traffic.

Document version: 1.0
Compatible with GUI: webgui-v1.3.0
Schema: fso-gw-stats/2
Last updated: 2026-04-25